Simulation of Pyridine–Water Separation via Heterogeneous Azeotropic Distillation

Mr. Ivica MARJANOVIC

UPMC – Université Pierre et Marie Curie, Paris – CAPGEMINI, Paris

Background & Description:

This project explores the simulation of a pyridine–water separation process using heterogeneous azeotropic distillation with Aspen HYSYS V11. The system includes two distillation columns designed to recover high-purity pyridine and water, using toluene as an entrainer.

The feed mixture forms a homogeneous azeotrope (pyridine–water), and the entrainer forms a heterogeneous azeotrope (water–toluene), enabling phase separation. The distillate of the first column is expected to separate into two immiscible liquid phases.

In this simulation, however, the decanter (V-100) was not successfully implemented in HYSYS, and the phase split was approximated manually.

Process Overview:

- Simulation Tool: Aspen HYSYS V11

- Thermodynamic Model: NRTL

- Feed Composition: ~77 mol% water, 23 mol% pyridine

- Entrainer: Toluene

Key Process Units:

- Distillation Column DC-01

  - Stages: 14

  - Feed Stage: 6

  - Reflux Ratio: 1.27

  - Bottoms Product: Pyridine (≥ 99.9 mol%)

  - Distillate: Water + Toluene (phase split approximated)

- Phase Split (V-100)

  - No decanter unit was successfully simulated.

  - Phase separation was assumed conceptually, based on literature behavior.

- Distillation Column DC-02

  - Stages: 10

  - Feed Stage: 6

  - Reflux Ratio: 0.5

  - Light Key in Bottoms: Water (0.999)

  - Heavy Key in Distillate: Pyridine (0.005)

  - Bottoms Product: Water (≥ 99.9 mol%)

Results & Limitations:

- The simulation achieved:

  - High-purity pyridine recovery in DC-01

  - High-purity water recovery in DC-02

  - Pyridine loss reduction through recycling

However, the decanter unit (V-100) could not be properly modeled in Aspen HYSYS. As a result, the liquid–liquid phase separation step was only approximated manually, without a rigorous simulation of equilibrium behavior.

This limitation prevents the simulation from being considered fully reliable or complete, especially for evaluating the recycle stream composition and dynamics.

Flowsheet

Results:

| Master Property Tableau |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Object | S-01-bis | S-02 | S-03 | S-11 | S-12 | S-13 |  |  |
| T (°C) | 25.00 | 118.6 | 113.9 | 25.00 | 93.47 | 99.45 |  |  |
| P (bar) | 1.013 | 1.115 | 1.115 | 1.013 | 1.013 | 1.013 |  |  |
| Molar Flow (kgmole/h) | 1.116e+08 | 1.800e+04 | 1.116e+08 | 1000 | 426.3 | 573.7 |  |  |
| Mole Fract (H2O) | 0.0022 | 0 | 0.0022 | 0.9000 | 0.7667 | 0.9990 |  |  |
| Mole Fract (Pyridine) | 0.7908 | 0.9992 | 0.7908 | 0.1000 | 0.2333 | 0.0010 |  |  |
| Mole Fract (Toluene) | 0.2070 | 0.0008 | 0.2071 | 0 | 0 | 0 |  |  |

Nb : Insignificant quantities (<0) of components in the product streams can be considered negligible, effectively approximating them to zero without impacting the overall process results or conclusions (see flowsheet).

References: Wu, Y.-C., & Chien, I.-L. (2009). Design and Control of Heterogeneous Azeotropic Column System for the Separation of Pyridine and Water. Industrial & Engineering Chemistry Research, 48(23), 10564–10576.